概述

Node是基于Arduino的物联网节点平台。该平台配合SeeedStudio的Grove电子模块系列和Xbee等通讯模块，用户可以快速地组建智能家居网络，并实现一定逻辑控制。使用Node平台来实现一个智能家居系统，例如远程人体监测报警、远程测距等，将变得非常方便和容易。

Node分为Atom.Node和Cloud.Node。

Atom主要用于局域网的通信。Atom通过Xbee或RFbee等无线模块进行相互通信，通信可靠，通讯距离高达100m；在配置上，用户使用SeeedStudio公司开发的基于Android平台的App，使用可见光通信的方式对Atom进行配置，配置过程简单直观；在使用上，Atom可以被单独使用来实现简单的IFTTT功能，也可以配对使用以实现远程无线控制和监控功能。

Cloud配备Xbee接口以及Wi-Fi功能，所以Cloud除了可以与Atom通信之外，也可以与互联网进行通信。配合Cloud，用户可以很方便的将Atom采集到的信号数据等上传至云端，或从云端对Atom进行简单控制。这将是一个很方便易用的智能物联网解决方案。

整体架构

图 1-1 Node工作示意图

上图反映了Cloud.Node是如何工作的，该架构有三个主要的部分：

- IoT Server:存放传感器数据的服务器，它们同时可以向你的设备发送一些命令，一些公共的服务器例如www.yeelink.com， www.xively.com

- Gateway: 传感器并不能直接跟IoT Server通信，需要通过一个网关。Cloud.Node就是扮演这个角色，当Cloud.Node接收到传感器节点的数据后，通过TCP/IP协议把数据发送到IoT Server

- Nodes: Atom.Node，各种各样的Sensor或Actuator

硬件组成

图 1-2 Cloud硬件组成

Cloud的硬件由两部分构成，Dragino以及插在上面的daughter board。Dragino与daughter board通过Uart以及SPI通信。

Daughter board上留有Xbee的接口，可以接Xbee，RFBee等通信模块与Atom.Node通信。Daughter board负责管理Node网络各Atom的管理，把数据通过Dragino与云端联系。

Dragino是一块基于OpenWrt的主板。

Daughter board通信接口

Daughter board通过串口向Dragino发送命令，为了便于管理和扩展，定义了以下的通信格式：

| START | DATATYPE | SPACE | DATA | END |
| --- | --- | --- | --- | --- |

START: 		“ss”

DATATYPE: 	1: ADD NODE; 	2: POST DATA;  3: DELETE NODE

SPACE:		a space between DATATYPE and DATA

DATA:		Valid Data

END:		“gg”

命令

添加一个设备

当有一个新的Atom.Node加入网络时，需要添加一个设备，发送以下命令可以完成这个功能。

Format:		“ss1 NODE_ID, SENSOR_ID, ACTUATOR_IDgg”

Function:		Create a Node with Specify SENSOR_ID and ACTUATOR_ID

Example: 	“ss 1 45,2,201gg”

上面的Example创建了一个device_Id为45，sensor_Id为2，actuator_Id为201的设备。

Once MS12 get this command from its UART port, MS12 will:

当Dragino接收到以上的命令后，会添加一个设备到设备管理器中，可以通过网页配置端的Devices List看到：

推送数据

Format:		“ss2 DEVICE_ID,VALUEgg”

Function:		推送一个传感器数值到 IoT server

Example:	“ss2 45,30gg”

删除一个设备

Format:		“ss3 DEVICE_IDgg”

Function:		删除一个设备

Example: 	“ss3 45gg”

删除device_id为45的设备

Daughter board的程序

Daughter board的程序主要负责：

- 接收Node网络的RF数据

- 管理Node网络各节点的数据

- 发送命令进行添加节点，数据推送及删除节点

软件流程图

图 3-1 程序流程图

主要函数接口

添加一个设备

/***********************************************************************************************

** Function name:           postDta to yeelink

** Descriptions:            return 1: ok 0: nok

***********************************************************************************************/

void NodeManage::yeelinkAdd(unsigned char idNode, unsigned char idSensor, unsigned char idActuator);

数据推送

/************************************************************************************************* Function name:           postDta to yeelink

** Descriptions:            return 1: ok 0: nok

***********************************************************************************************/

void NodeManage::yeelinkPost(unsigned char idNode, unsigned int psDta);

如何访问Dragino

通过网页配置端

网页输入192.168.255.1，即可登录到Dragino的网页配置端。

图 4-1 Dragino的网页配置

通过SSH

Dragino上跑了一个嵌入式Linux操作系统:OpenWrt。可以通过SSH登录：

图 4-2 通过SSH登录Dragino

The SSH access for Dragino is:

IP address: 172.31.255.254 on LAN port.

Username: root

Password: root

SSH access password can be changed by type below commands:

root@dragino-751aff:~# passwd

Changing password for root

New password:

Retype password:

Password for root changed by root

Notice: for security reason, it is recommend that you change the SSH access password after first log in.

网页配置界面

Sensor Settings

General Settings:

- Save Sensor Data to Local File: Enable/Disable local logger for sensor data from UART port.

- Sensor Data Location: File to store the sensor data. Can be changed in /etc/config/sensor

- Debug:	Enable/Disable Debug. Debug info will show on Syslog page

- Enable Custom Code: Enable/Disable Custom Code.

- Custom Code: use Linux commands to process sensor data. The Custom commands will be executed once there is new incoming string from UART port. Developer can use macro [RAW_DATA] to get the incoming string

Example:

echo [RAW_DATA] >> tem.log; atftp -p -r tem.log 192.168.1.2; rm tem.log

This command will forward the RAW_DATA to tftp server with file name tem.log

Note: Due to security reason, Enable Custom Code and Custom Code options are set to invisible by default. They can be set to visible by set the option “show_custom_code” to “1” in file /etc/config/sensor

IoT Server Configure:

- Enable IoT Service:		Disable/Enable IoT Service

- IoT Service:			Choose Service Provider

- Internet Connection:	Show the connection to the service provider

- API Key:				API key from service provider

- Title:				Default title prefix when create nodes on the IoT server. If leave this blank, the default tile will be set to host name of the MS12.

Devices List:

Shows the Node Connected to Dragino MS12.

Wi-Fi Settings

Wi-Fi Settings:

- Network Name(ESSID): 	Input your wifi router SSID

- Encryption:				Encryption method used by your router

- Key:						Encryption Key of your wifi network

Network Interfaces

Internet Connection

WAN Interface: 		Interface for internet access, either WIFI or RJ45 port

Protocol:				Way to get IP: Manual, DHCP, PPPoE

Clamp Segment Size:	Normally don’t use this

Local Network – Network distributed by MS12

LAN interface:			Interface for local network, either WIFI or RJ45 port

IPv4-Address:			IP address for the interface used for local network.

IPv4-Netmask:		Net mask for this local network

IPv4-Gateway:		Gateway for this local network, default is the IP address set up above.

DNS-Server:			DNS for this local network.

System Status

System status page.

DDNS settings

Dynamic DNS allows you access/control the Dragino & Dragrove from other location even you don’t have a fix IP Address.

Enable DDNS Service: Enable/Disable DDNS service

DDNS Service Provider: choose your service provide here

Domain: the hostname provide by your DDNS service provider.

Username: Username of your DDNS service

Password: Password of your DDNS service

Source of IP-Address: Where to look for your external IP address. You can choose:

Network: Set external IP according to your network interface info, e.g. wan, lan

Interface: Set external IP according to your hardware network interface info: eth0 , ath0.

URL: Set external IP according to URL info, for example, you can set it to http://www.whatismyip.com/automation/n09230945.asp so the Dragino will connect to this url and get its external IP. It is used when Dragino have a private IP address in its wan port.

Check for changed IP: how often to check if it needs to update its IP to ddns service provider.

Force update: how often to force an update to DDNS service provider

Upgrade Firmware

The latest firmware of Dragino can be found in below link:

http://www.dragino.com/downloads/index.php?dir=MS12/firmware/

Valid upgradable firmware via GUI has a suffix combined.img. For IoT application, choose the firmware in under IoT directory.

- REFERENCE

- www.seeedstudio.com	: Dragrove vendor, more info about Dragrove and its development kit can be found here.

- www.openwrt.org: Embedded linux used in Dragino.

- wiki.dragino.com: General software/hardware design info for Dragino MS12

- www.yeelink.com: A public IoT RESTful server used in China.

- https://github.com/reeedstudio/Cloud_Dragrove: Daughter board firmware code

- https://github.com/reeedstudio/Cloud_Dragino_Firmware： Dragino Firmware